C180 -185L High Heat Sterilization CO2 Incubator Stainless Steel

Use

For static culture of cell , it’s high heat sterilization CO2 incubator with HEPA filter.

Performance

- 4.3 inch LCD touch screen, intuitive control easy to operate.

- 304 frosted stainless steel arc angle integrated inner cavity, beautiful and easy to clean.

- Internal six-sided direct heating system ensures temperature uniformity.

- HEPA air filtration continuously removes particulates for Class ISO 5 level quality conditions.

- HEPA filter and IR sensor are easily removed.

- 140°C high heat sterilization ensure internal clean.

- Circulating airflow technology ensures high consistency of culture conditions in the chamber.

- Clean humidified pan to maintain culture humidity environment.

- After stable operation, if the temperature fluctuation exceeds the set value plus or minus overtemperature alarm deviation, audible alarm activates.

- Audible alarm activates if outer door is opened over 1 minute.

- Historical data can be viewed in real time，and can be exported by the USB interface.

Technical Details

| Model | C180 |  |
| --- | --- | --- |
| Control interface | 4.3 inch LCD touch screen |  |
| Temperature control mode | PID control mode |  |
| Temperature control range | Ambient +5°C~60°C |  |
| Temperature display resolution | 0.1°C |  |
| Temperature fluctuation | ±0.1°C |  |
| Temperature field uniformity | ±0.3°C @37°C |  |
| Power of heating | 900W |  |
| Timing function | 0-999.9hours |  |
| Internal size | L541 x W508 x H681mm |  |
| External size | L660 x W640 x H1000mm |  |
| Volume | 185L |  |
| Principle of CO2 sensor | IR sensor detection |  |
| CO2 control range | 0-20% |  |
| CO2 display resolution | 0.10% |  |
| CO2 supply | 0.05~0.1MPa is recommended |  |
| HEPA filtering | ISO 5 level, 5 minutes |  |
| Sterilization | 140°C High heat sterilization |  |
| Maximum expansion | Up to 2 units can be stacked |  |
| Ambient temperature | 5°C~ 40°C |  |
| Power supply | 115V~230V±10%, 50~60Hz |  |
| Weight | 108kgs |  |

Optional accessories

HEPA filter

Access Port filter